Robo Soccer

1.Chassis

Chassis length formula

Chassis length = motor length * Wheel Diameter + battery Space

Chassis Width Formula

Width =wheel to wheel +1-2cm

Tentative calculation for wheel diameter motor, RPM, linear speed

Linear speed(m/s) =RPM *pi*wheel-diameter/60

Example: -

With setup (37GB,500 RPM ,85 mm wheel)

Speed =approx. 2.224m/s

2. Recommended Motor Type & Motor Drive

37 GB gear DC Motor-500 RPM

Why 37 GB gear DC motor?

A 37GB gear motor is very commonly used in robotics because it gives a perfect balance

1.High Torque

The “GB” stands for Gear Box.
The gearbox increases torque — which means your robot can:

- Move smoothly even on uneven surfaces

2. Compact Size

37GB motors are small but powerful.

- 37mm diameter

- Easy to mount

- Not too heavy → good for speed + endurance

3.Durable & Metal Gearbox

The gears are metal, so they won’t break easily during load or collision.

4. Wide Voltage Options

You can get them in 6V, 12V, or 24V versions.
This gives flexibility for battery selection and speed requirement.

5. Many RPM Choices

You can choose the gearbox ratio depending on what your robot needs:

| RPM | Good For |
| --- | --- |
| 10–60 RPM | Very high torque lifting bots |
| 100–300 RPM | Combat robots, pushing |
| 500+ RPM | Fast racing bots |

So, one motor model fits different robot types!

6. Cheaper Than Brushless Motors

Good performance at low cost → best for students and competitions.

Reference Link: - https://www.youtube.com/watch?v=gxoFwbduTOc

Recommended Motor Drive/ESCs: -

It is controlled using a motor drive (BTS7960, Cytron)

Why you should use BTS7960?

1. High Current Capacity

BTS7960 can handle 43A peak current.

Your 37GB motor may draw 5-10A stall current, so BTS7960 has more than enough margin → safe, strong, reliable.

2. H-Bridge Built-In (Forward + Reverse) It has a complete H-bridge circuit, so your robot can:

Move Forward

Move Backward

Stop

Turn, by controlling motors independently

3. Control System

Transmitter & Receiver